MODEM-G2 specifications

Revision: 1.3

Date:  29/01/98

Reference: WM02sp13.doc

|  |  |  | DOCUMENT | AMENDMENTS |
| --- | --- | --- | --- | --- |
| ISSUE | DATE | STATUS | AUTHOR | COMMENTS |
| 1.0 | 20/04/98 | P | CGi | Creation |
| 1.2 | 21/01/99 |  | CGi | Sticker, Packaging, Accessories |
| 1.3 | 29/01/98 |  | CGi | Form |

1.	Introduction	1

1.1.	Objective of the document	1

1.2.	Reference documents	1

2.	Basic offer and accessories	1

2.1.	Basic offer	1

2.1.1.	Contained	1

2.1.2. Packaging	1

2.1.3.	User manual	2

2.2.	Accessories	2

2.2.1.	Cordons	2

2.2.2.	Headset	2

2.2.3.	Antenna	2

2.2.4.	Power supply	2

2.3.	Options	2

2.3.1.	Car Kit	2

3.	Marketing	3

3.1.	Product marketing research	3

3.2.	The types of markets	3

3.5.	Evolution	3

4.	Product management	4

4.1.	WAVECOM reference	4

4.2.	Markings by labels	4

4.2.1.	Product label	4

4.2.2.	Production sticker	4

4.2.3.	CE and IMEI sticker	4

4.2.4.	Label packaging	4

5.	Context specifications	5

5.1.	Normative specifications	5

5.2.	Environment specifications	5

5.2.1.	Climatic and mechanics environment	5

5.2.2.	Electric environment	6

5.3.	Physical characteristics	7

6.	General description	8

6.1.	Presentation	8

6.2.	Mechanical philosophy retained	9

6.3.	Connectors definition	10

6.3.1.	Connector location	10

6.3.2.	Connectors description	10

6.5.	Electrical characteristics	12

6.6.Electrical functions	13

6.6.1.	Functional schematic	13

6.6.2.	Filtering	14

6.6.3.	ESD protection	14

6.6.4.	Voltage surge protection	14

6.6.5.	Regulation	14

6.6.6.	RS232 Interface	14

6.6.7.	Amplification	14

6.6.8.	Other function	14

7.	Accessories description	15

7.1.	Headset	15

7.2.	Cordon examples	15

7.2.1.	Cordon RS232/AUDIO(head set) :	15

7.2.2.	Cordon RS232/AUDIO(Car Kit)  :	15

7.2.3.	Power supply cordon :	16

7.2.4.	Cordons construction (RS232/Audio head set):	16

# 1.	Introduction

## 1.1.	Objective of the document

This document talks about the specifications of the second generation GSM modem. Modem product is referenced like this: WMO2-GXXXX according to standards (WMO2-G900 : GSM  ; WMO2-G1800 : DCS  ; WMO2-G1900 : PCS). All these modem products are based on WISMO concept, it means each modem include a WISMO1B-Gxxxx module .

In this document you will find, on the one hand the description of the basic modem offer and on the other hand several accessories.

## 1.2.	Reference documents

- ETS 300 019		Environmental conditions and environmental tests for telecommunications equipment.

- Plan de qualification interne WAVE	version 1.1.

- SAE J1113		Transients Voltage Suppression in Automotive Vehicle.

# 2.	Basic offer and accessories

## 2.1.	Basic offer

### 2.1.1.	Contained

Basic offer include the following elements:

- Modem

- Mechanical fixation (holding bridle)

- Power supply cable + fuse

- User manual

### 2.1.2. Packaging

The Basic offer set is presented in an unique conditioning. External dimensions close to the following values:

70mm (width) x 60mm (height) x135mm (length).

This is a carton box. It is build in small waves which are covered with a thin film of white ice-cold paper.

### 2.1.3.	User manual

User manual is realised in a size close to the following values:

105mm (width) x 148,5 mm (height) (1/2 A5).

It contains twenty two pages except the coverage.

## 2.2.	Accessories

### 2.2.1.	Cordons

Two cordons could be proposed:

- serial link and audio cable (Y cordon) for a low power audio solution (for example : head set, phone receiver, ...),

- serial link and audio cable (Y cordon ) for Car Kit option.

### 2.2.2.	Headset

Headset allows an audio hand free function. This last one integrates a weak power speaker phone and a microphone.

TBD  :standard products

### 2.2.3.	Antenna

Two solutions could be proposed:

- a standard deported cellular phone antenna,

- a WAVECOM designed short antenna.

### 2.2.4.	Power supply

An AC(220V~)/DC(12V/2A) converter allows a power network connection.

## 2.3.	Options

### 2.3.1.	Car Kit

Thanks to an audio power amplifier, a microphone fixed on vehicle sun visor and a loud speaker fixed on the vehicle dashboard, anybody can obtains  a hand free embarked solution.

Speaker (8) and directional microphone are supplied with this option.

# 3.	Marketing

## 3.1.	Product marketing research

Market research.

Competitor analysis.

## 3.2.	The types of markets

Modem plug in to a PC,

Alarm,

Vehicle Localisation,

Metering.

## 3.5.	Evolution

Dual band, Dual Mode ?

Micro controller option.

# 4.	Product management

## 4.1.	WAVECOM reference

WMO2-GXXXX (WMO2-G900, WMO2-G1800 , WMO2-G1900).

## 4.2.	Markings by labels

### 4.2.1.	Product label

This label is built with anodise aluminium. This one, has a blue silk screen treatment (marking : aluminium colour) and is located on the upper convex mechanical part. It supports the WAVECOM logo. Dimension: 96 x10 mm.

### 4.2.2.	Production sticker

This label is located on the back of the product and contains CE marking (in order to improve the productivity on assembly line, this marking will be printed matter or silk screened in the laser). Dimension  : 60 x 9.5 mm.

0165

Made by Wavecom

### 4.2.3.	CE and IMEI sticker

This label placed in the back of the product, it contains the following information  : REF PROD (WMO-GXXX), REF WISMO   WM1B-GXXX, PN (WMXXXXX) in clear and IMEI number in barre code. This last one integer the product serial number. Dimensions  : 60 x 9.5 mm.

REF PROD : WMO2-GXXX-xxx-xx

PN : WMxxxxx

IMEI

### 4.2.4.	Label packaging

This label is put on the product box. Define the contents.

# 5.	Context specifications

## 5.1.	Normative specifications

Product is compliant with specification ETSI Phase II.

## Environment specifications

### 5.2.1.	Climatic and mechanics environment

The following figure shows environment standard constraints:

### 5.2.2.	Electric environment

The following figure sum up electrical constraints in an automotive environment:

| Length of transient | Cause | Energy capability Voltage Amplitude | Possible frequency of application |
| --- | --- | --- | --- |
| Steady state | Failed Voltage Regulator | oo + 18 V | Infrequent |
| 3 - 5 minutes | Jump start with 24 V battery | oo +/- 24 V | Infrequent |
| 200ms to 400ms | Load dump - i.e., disconnection of battery while at high charging rates | 10 J 125V | Infrequent |
| < 0.32 s | Inductive Load Switching Transient | <1 J -300V to +80V | Often |
| < 0.20 s | Alternator Field Decay | <1 J -100V to -40V | Each Turn-Off |
| 90ms | Ignition Pulse, Battery Disconnected | <0.5 J 75V | <500Hz Several Times in vehicle Life |
| 1ms | Mutual Coupling in Harness (Note) | <1 J <200V | Often |
| 15µs | Ignition Pulse, Normal | <0.001 J <3V | <500Hz Continuous |
|  | Accessory Noise | <1.5V | 50 Hz to 10 KHz |
|  | Transceiver Feedback | 20mV | R.F. |

Note : These transients may be present on any wire in the vehicle.

## 5.3.	Physical characteristics

The physical characteristics are the following ones:

| Physical characteristic | Qualification | Comments |
| --- | --- | --- |
| Dimension Absolute maximum dimension | 98x54x25 mm 110x54x25 mm | Without the connectors quotations . |
| Weight | < 140 g |  |
| Volume | 13.23 cm3 |  |
| Case |  | Aluminium profile |

5.4.	Basic services

Functions of this modem are the following ones:

|  | GSM | DCS/PCS |
| --- | --- | --- |
| Standard | 900 MHz. Class 4 (2W). GSM phase 2. | 1800 MHz or 1900 MHz Class 5 (1W) GSM phase 2. |
| Interface | Serial interface RS232 V.24/V.28 AT command set based on V.25ter and GSM 07.05 & 07.07. Auto-bauding function between 2400 bits/s and 19200 bits/s No auto-framing available | Serial interface RS232 V.24/V.28 AT command set based on V.25ter and GSM 07.05 & 07.07. Auto-bauding function between 2400 bits/s and 19200 bits/s No auto-framing available |
| SMS | Mobile Originated (MO) and Mobile Terminated (MT). Mode Text & PDU point to point. Cell broad cast. In accordance with GSM 07.05 | Mobile Originated (MO) and Mobile Terminated (MT). Mode Text & PDU point to point. Cell broad cast. In accordance with GSM 07.05 |
| Data | Asynchronous 2400, 4800, 9600 bits/s. Transparent and Non Transparent mode In Non Transparent Mode: 300, 1200, 1200/75 bauds. Mode 3.1 KHz (PSTN) and V110 (ISDN) | Asynchronous 2400, 4800, 9600 bits/s. Transparent and Non Transparent mode In Non Transparent Mode: 300, 1200, 1200/75 bauds. Mode 3.1 KHz (PSTN) and V110 (ISDN) |
| Fax | 2400/4800/7200/9600 bits/s, GSM teleservice 62 in Transparent Mode. Class 1. Group 3 compatible. | 2400/4800/7200/9600 bits/s, GSM teleservice 62 in Transparent Mode. Class 1. Group 3 compatible. |
| Audio | FR et EFR operation 1 : Head Set 2 : Car Kit (in option) | FR et EFR operation 1 : Head Set 2 : Car Kit (in option) |

# 6.	General description

## 6.1.	Presentation

Modem mechanical case is built from an aluminium profile ended by two stoppers at these extremities. All input/output links are established by three connector placed at the two extremities of the profile. No cordon is fixed to the case. The SIM card (micro-SIM) is put on an extract drawer. A red indicator specifies the functioning mode of the product.

## Mechanical philosophy retained

## 6.3.	Connectors definition

### 6.3.1.	Connector location

### 6.3.2.	Connectors description

We apply the following constraints in the connector choice:

- size,

- mechanical,

- electrical performances,

- Industrialisation (surface assembly choice).

In order to extract or insert the Micro SIM card, each user has to press with a sharp element (a pen for example) the SIM holder ejector.

If this procedure is no respected, the SIM holder could be destroyed.

| Function connector | Type | Pining | Drawing | Ref. supplier | opposite connector examples |
| --- | --- | --- | --- | --- | --- |
| RF connector | SMA |  |  | RADIALL : R284310085 | RADIALL : R125073 |
| 1 2 3 4 5 6 7 8 9 10 15 14 13 11 12 Serial link AUDIO link BOOT RESET | SUB D High Density (15 pts) | 1 DCD (CT109) 2 TX (CT103) 6 RX (CT104) 7 DSR (CT107) 8 DTR (CT108/2) 9 GND 11 CTS (CT106) 12 RTS (CT105) 13 RI (CT125) 4 MICROPHONE (+) 5 MICROPHONE (-) 10 SPEAKER (+) 15 SPEAKER (-) 3 BOOT 14 RESET |  | JST : KSEY-15S-3B6L18-13 | ITT CANNON : ZDEA-15P-SB or JST : KEC-15P with contact JK-SP2143 |
| 4 3 2 1 Power Supply connector | Micro-Fit (4pts) | 1 V+BATTERY 2 GROUND 3-4 AUXI |  | MOLEX : 43045-0409 | MOLEX : 43025-0400 |
| SIM holder |  |  |  | MOLEX : 52828-0611 | MICRO SIM |

## 6.5.	Electrical characteristics

The following figure summarises electrical characteristics defined in input/output for all external connections.

| Parameters | MIN | TYP | MAX | UNIT | Comments |
| --- | --- | --- | --- | --- | --- |
| Power supply : Input supply voltage Input supply voltage with Car Kit option Input peak supply current @5V*/6V** Input average supply current @5V*/6V** in communication mode Input average supply current @5V*/6V** in idle mode (paging period 480ms) Input average supply current @5V* in idle mode with auto-shutdown function*** | 5*/6** | 13,5 13,5 | 32 18 2,5*/0,9** 450*/200** 35 10 | V V A mA mA mA | GSM or DCS/PCS GSM or DCS/PCS GSM or DCS/PCS |
| Serial link : RS232 |  |  |  |  | standard |
| Audio (head set) : microphone input current @2V/2K absolute microphone input voltage speaker output current 150 //1nF absolute speaker impedance |  | 0,5 16 | 100 32 | mA mVpp mA  |  |
| SIM | 3 |  | 5 | V |  |

* 	only GSM

** 	only DCS/PCS

*** auto shutdown function could be activate if the serial link leads a non hardware flux (CTS/RTS non used). This hardware function will not be available with the first products.

## 6.6.Electrical functions

6.6.1.	Functional schematic

POWER SUPPLY REGULATION

Main regulation

Vcc modem

Power supply

Filtering

V Protection

RS232 INTERFACE

Regulation

Vcc modem

RS232 link (WISMO module)

RS232 link (application)

ESD Protection

Filtering

RS232 interface

AUDIO INTERFACE : Head Set

Micro Module

MICRO

Speaker Module

Protection ESD

Filtering

Filtering

SPEAKER

Protection ESD

AUDIO INTERFACE : Car Kit (in option)

Micro Module

MICROPHONE

Speaker Module

ESD Protection

Filtering

SPEAKER

Amplification

Filtering

### 6.6.2.	Filtering

Filtering guarantees:

- EMI/RFI protection in input and output,

- signal smoothing.

Components retained are on the one hand Low Pass Filter (LC filter architecture) and other one power inductor.

### 6.6.3.	ESD protection

This function protects all input/output against the electrostatic surges.

Components which compose this function will be sized according to, the power of the electrostatic surges apply on the connectors and the normal functioning voltage scale. Components retained are TRANZIL diodes (TVS diode : Transient Voltage Suppression diode).

### 6.6.4.	Voltage surge protection

This function allows a protection against, the voltage surges, the reverse polarity and electrostatics surges on the power supply line.

### 6.6.5.	Regulation

Main regulation is made with a DC/DC converter in order to supply power at all functions (STEP-DOWN CONVERTER is the name of montage).

### 6.6.6.	RS232 Interface

This function adapts voltage levels between the WISMO module and external world .

### 6.6.7.	Amplification

This function is obtains with an audio power amplifier.

### 6.6.8.	Other function

Red indicator. This function shows three operating modes:

the red indicator light up and light down fast in the transmission mode,

the red indicator light up and light down slowly in the idle mode,

the red indicator is light up continuous in the network search mode

# 7.	Accessories description

## 7.1.	Headset

References of this kit have to be specified.

## 7.2.	Cordon examples

### 7.2.1.	Cordon RS232/AUDIO(head set) :

Flat cable   4 wires  :

Wire constitution : tinned copper 7x0.15 mm (26AWG)

Maximum operating temperature -20°C/+85°C

Modular Jack

4P/4C

SUB-D High Density 15 pts mâle + braid of shield soldered on SUB D body + cover.

SUB-D 9 pts femelle

+ braid of shield soldered on SUB D body + cover.

Cable length : 1 m .

Shielded cable   9  wires + braid of shield :

Wire constitution : tinned copper 7x0.2 mm

Wire constitution : 7x 0.15 ( 26AWG)

Maximum operating temperature: -20°C/+85°C.

### 7.2.2.	Cordon RS232/AUDIO(Car Kit)  :

Jack audio mono 2,5 mm female (microphone)

Cable   2x2 wires  :

2 Wires constitution (speaker) : tinned copper 7x0.2 mm (24AWG)

1 conductor with shield (microphone)

Maximum operating temperature -20°C/+85°C

phono connector RCA female (speaker)

SUB-D High Density 15 pts mâle + braid of shield soldered on SUB D body + cover.

Cable length : 1 m .

SUB-D 9 pts femelle

+ braid of shield soldered on SUB D body + cover.

Shielded cable   9  wires + braid of shield :

Wire constitution : tinned copper 7x0.2 mm

Wire constitution : 7x 0.15 ( 26AWG)

Maximum operating temperature: -20°C/+85°C.

### 7.2.3.	Power supply cordon :

Cable : 1 wire.

Ame : tinned copper 24x0.2 mm

Section : 0.75 mm²

Maximum operating temperature: -20°C/+85°C.

MICRO FIT connector 4 pts (MOLEX: 43025-0400).

Contacts (x2) (MOLEX: 43030-0007.

Side view

Black

Fuse 5x20 mm. (2,5 A)

Red

Tinned wires

Cable length : 1 m .

### 7.2.4.	Cordons construction (RS232/Audio head set):

Assembly example

| SUB-D 9CTS | Connection | SUB D 15CTS (RS232) | Signal name |
| --- | --- | --- | --- |
| 1 (White) |  | 1 (White) | DCD (CT109) |
| 2 (Grey) |  | 6 (Grey) | RX (CT104) |
| 3 (Violet) |  | 2 (Violet) | TX (CT103) |
| 4 (Bleu) |  | 8 (Bleu) | DTR (CT108/2) |
| 5 (Black) |  | 9 (Black) | GND |
| 6 (Orange) |  | 7 (Orange) | DSR (CT107) |
| 7 (Yellow) |  | 12 (Yellow) | RTS (CT105) |
| 8 (Brown) |  | 11 (Brown) | CTS (CT106) |
| 9 (Green) Top view |  | 13 (Green) | RI (CT125) |
| Modular Jack 4P/4C 1 | Connection | SUB D 15CTS (AUDIO) |  |
| 1 2 3 4 1 (Yellow/White) 2 |  | 4 (Yellow/White) | microphone(+) |
| 2 (Red/White) 3 |  | 10 (Red/ White) | speaker(+) |
| 3 (Green/White) 4 |  | 15 (Green/White) | speaker(-) |
| 4 (Black/White) |  | 5 (Black/White) | microphone(-) |
| Flying wires | Connection | SUB D 15CTS (Boot/Reset/GND) |  |
| (Pink) |  | 3 (Pink) | BOOT |
| (Bleu/White) |  | 14 (Bleu/White) | RESET |
| (Black) |  | 9 (Black) | GND |